Case Study 4: Demonstration of Network Communication Models (OSI / TCP-IP) Using a Network Simulation Tool

# Aim

To demonstrate OSI and TCP/IP network communication models using a network simulation tool.

# Objective

Configure a simple network topology.

Observe packet delivery and address resolution.

Map OSI layers to TCP/IP layers.

Understand basic network troubleshooting.

# Selected Tool

Cisco Packet Tracer or a comparable educational network simulator.

# Network Topology

Two PCs are connected to a switch. An optional router can be added to demonstrate communication between different IP networks.

| Device | Example IPv4 Address | Subnet Mask | Purpose |
| --- | --- | --- | --- |
| PC-A | 192.168.1.10 | 255.255.255.0 | Source host |
| PC-B | 192.168.1.20 | 255.255.255.0 | Destination host |
| Switch | Not required for basic switching | N/A | Forwards Ethernet frames using MAC addresses |

# Procedure

Create two PCs and connect them to a switch.

Configure the example IP addresses and subnet masks.

Send an ICMP ping from PC-A to PC-B.

Use simulation mode to observe ARP resolution and ICMP echo request/reply packets.

Inspect protocol layers and source/destination addressing.

Optionally use different subnets and configure a router and default gateways.

# OSI and TCP/IP Model Mapping

| OSI Layer | TCP/IP Layer | Example Function / Protocol |
| --- | --- | --- |
| Application | Application | HTTP, DNS, application data |
| Presentation | Application | Data representation, encoding, encryption concepts |
| Session | Application | Session establishment and management concepts |
| Transport | Transport | TCP/UDP ports and end-to-end transport |
| Network | Internet | IP addressing and routing |
| Data Link | Network access / Link | Ethernet frames and MAC addressing |
| Physical | Network access / Link | Signals, cables, connectors, and bits |

# Expected Observations

PC-A can ping PC-B when configuration is correct.

ARP may resolve an IPv4 address to a local MAC address.

The switch forwards frames using its MAC address table.

Routing is needed for traffic between different IP networks.

Packet inspection shows headers and packet movement across devices.

# Troubleshooting

| Problem | Possible Cause | Remedy |
| --- | --- | --- |
| Ping fails | Incorrect IP address or subnet mask | Verify host addressing |
| No link | Cable or interface problem | Check links and interface status |
| Cross-subnet communication fails | Missing gateway or route | Check default gateway and router routes |
| ARP repeats | Destination unreachable or mapping unavailable | Verify host and local network |

# Results

The simulation demonstrates how layered communication models correspond to network traffic and how correct addressing and topology affect delivery.

# Conclusion

The OSI model provides a useful framework for understanding and troubleshooting network communication, while TCP/IP represents the protocol suite used by modern networks.